| PACIFIC COAST ELECTRIC Licensed Electrical Contractor ¦ CA Lic. #C10-892341 (408) 555-0147 ¦ info@pcelectricsj.com ¦ San Jose, CA 95123 | ESTIMATE Estimate No: EST-2025-001 Date Issued: March 2, 2025 Valid Until: April 1, 2025 |
| --- | --- |

| BILL TO Name: Jennifer & Mark Rossi Phone: (408) 555-2987 Email: j.rossi@gmail.com Address: 4821 Blossom Hill Rd, San Jose CA 95118 | JOB SITE / PROJECT Service Address: 4821 Blossom Hill Rd, San Jose CA 95118 Job Type: EV Charger Installation (Level 2 — 240V) |
| --- | --- |

SCOPE OF WORK Install one (1) dedicated 240V / 50A circuit for a Level 2 EV charger in the attached garage. Existing 200A main panel has available breaker space. Run 35 ft of 6/3 NM-B cable from panel to garage, install 50A double-pole breaker, mount a NEMA 14-50 outlet at customer-specified location on garage wall. Pull city permit. Test circuit and verify charger compatibility.

LINE ITEMS

| Description | Qty | Unit | Labor ($) | Materials ($) | Line Total ($) |
| --- | --- | --- | --- | --- | --- |
| 50A double-pole circuit breaker (Square D QO250) | 1 | ea | 0.00 | 65.00 | 65.00 |
| 6/3 NM-B copper wire (35 ft run) | 35 | ft | 0.00 | 87.50 | 87.50 |
| NEMA 14-50 flush-mount outlet + weatherproof cover plate | 1 | ea | 0.00 | 42.00 | 42.00 |
| Electrical boxes, connectors, staples, wire nuts | 1 | lot | 0.00 | 28.00 | 28.00 |
| Labor — panel work, wire run, outlet installation, testing | 5 | hr | 750.00 | 0.00 | 750.00 |
| City of San Jose electrical permit | 1 | ea | 0.00 | 150.00 | 150.00 |

|  | Labor Subtotal: $750.00 |
| --- | --- |
|  | Materials Subtotal: $222.50 |
|  | Permit & Inspection Fees: $150.00 |
|  | GRAND TOTAL: $1,122.50 |

TERMS & NOTES Price assumes no panel upgrade is needed. If inspector requires panel work, a separate estimate will be provided. Customer is responsible for purchasing the EV charger unit. This estimate covers installation only. This estimate does not include patching or painting drywall if wire run requires wall penetration. All work performed per NEC 2023 and City of San Jose electrical code requirements. This estimate is valid for 30 days from date of issue. Payment Terms: 50% deposit required to schedule. Balance due upon job completion. Acceptance: Signing below constitutes agreement to the scope and price above. Customer Signature: ___________________________________________ Date: ______________

| PACIFIC COAST ELECTRIC Licensed Electrical Contractor ¦ CA Lic. #C10-892341 (408) 555-0147 ¦ info@pcelectricsj.com ¦ San Jose, CA 95123 | ESTIMATE Estimate No: EST-2025-002 Date Issued: March 5, 2025 Valid Until: April 4, 2025 |
| --- | --- |

| BILL TO Name: David Okonkwo Phone: (408) 555-7741 Email: dokonkwo@yahoo.com Address: 2204 Meridian Ave, San Jose CA 95125 | JOB SITE / PROJECT Service Address: 2204 Meridian Ave, San Jose CA 95125 Job Type: Electrical Service Panel Upgrade — 100A to 200A |
| --- | --- |

SCOPE OF WORK Complete service upgrade from 100-amp to 200-amp electrical service. Replace existing Federal Pacific 100A panel with a new 200A Square D HomeLine main breaker panel (32-space, 64-circuit). Install new meter base, service entrance cable, and weatherhead. Coordinate temporary power shutoff with PG&E. Install 10 tandem breakers to match existing circuits. Pull city permit, schedule inspection, and restore all power. Estimated duration: 1 full business day.

LINE ITEMS

| Description | Qty | Unit | Labor ($) | Materials ($) | Line Total ($) |
| --- | --- | --- | --- | --- | --- |
| 200A Square D HomeLine main panel (32-space) | 1 | ea | 0.00 | 320.00 | 320.00 |
| 200A main breaker | 1 | ea | 0.00 | 85.00 | 85.00 |
| Square D tandem breakers (to match existing circuits) | 10 | ea | 0.00 | 180.00 | 180.00 |
| Meter base (200A, ringless) | 1 | ea | 0.00 | 95.00 | 95.00 |
| 4/0 aluminum service entrance cable (10 ft) | 10 | ft | 0.00 | 60.00 | 60.00 |
| Weatherhead, conduit, connectors, grounding rod + clamp | 1 | lot | 0.00 | 140.00 | 140.00 |
| Labor — full panel swap, meter base, service entrance, circuit transfer, testing | 8 | hr | 1,200.00 | 0.00 | 1,200.00 |
| City of San Jose permit + PG&E coordination fee | 1 | ea | 0.00 | 250.00 | 250.00 |

|  | Labor Subtotal: $1,200.00 |
| --- | --- |
|  | Materials Subtotal: $880.00 |
|  | Permit & Inspection Fees: $250.00 |
|  | GRAND TOTAL: $2,330.00 |

TERMS & NOTES PG&E will disconnect and reconnect power at no additional charge to customer. Typical scheduling is 2-5 business days. Price includes transfer of all existing circuits to new panel. Does not include adding new circuits. Existing wiring condition cannot be fully assessed until panel is open. Any required rewiring will be quoted separately. Federal Pacific Stab-Lok panels are known fire hazards — replacement is strongly recommended. This estimate is valid for 30 days from date of issue. Payment Terms: 50% deposit required to schedule. Balance due upon job completion. Acceptance: Signing below constitutes agreement to the scope and price above. Customer Signature: ___________________________________________ Date: ______________

| PACIFIC COAST ELECTRIC Licensed Electrical Contractor ¦ CA Lic. #C10-892341 (408) 555-0147 ¦ info@pcelectricsj.com ¦ San Jose, CA 95123 | ESTIMATE Estimate No: EST-2025-003 Date Issued: March 8, 2025 Valid Until: April 7, 2025 |
| --- | --- |

| BILL TO Name: Sofia & James Chen Phone: (408) 555-3391 Email: chenfamily.sj@gmail.com Address: 1087 Foxworthy Ave, San Jose CA 95118 | JOB SITE / PROJECT Service Address: 1087 Foxworthy Ave, San Jose CA 95118 Job Type: Kitchen Renovation — Electrical Upgrade |
| --- | --- |

SCOPE OF WORK Electrical upgrade for full kitchen renovation. Install 3 new dedicated 20A circuits for refrigerator, microwave, and dishwasher. Replace 6 standard outlets with GFCI outlets (code-required within 6 ft of sink/countertop water source). Add 2 new under-cabinet outlet strips for small appliances. Install 1 recessed LED fixture over island (homeowner supplying fixture). All new 12/2 NM-B wire. Pull permit.

LINE ITEMS

| Description | Qty | Unit | Labor ($) | Materials ($) | Line Total ($) |
| --- | --- | --- | --- | --- | --- |
| 20A dedicated circuit — refrigerator (12/2 NM-B, 20A breaker) | 1 | ea | 180.00 | 75.00 | 255.00 |
| 20A dedicated circuit — microwave (12/2 NM-B, 20A breaker) | 1 | ea | 180.00 | 75.00 | 255.00 |
| 20A dedicated circuit — dishwasher (12/2 NM-B, 20A breaker) | 1 | ea | 180.00 | 75.00 | 255.00 |
| GFCI outlet replacement (code-required locations) | 6 | ea | 270.00 | 120.00 | 390.00 |
| Under-cabinet outlet strip installation (2 locations) | 2 | ea | 120.00 | 80.00 | 200.00 |
| Recessed LED fixture — rough-in wiring + install (fixture by owner) | 1 | ea | 90.00 | 25.00 | 115.00 |
| Miscellaneous — wire nuts, staples, box extensions, faceplates | 1 | lot | 0.00 | 45.00 | 45.00 |
| City of San Jose electrical permit | 1 | ea | 0.00 | 150.00 | 150.00 |

|  | Labor Subtotal: $1,020.00 |
| --- | --- |
|  | Materials Subtotal: $495.00 |
|  | Permit & Inspection Fees: $150.00 |
|  | GRAND TOTAL: $1,665.00 |

TERMS & NOTES This estimate assumes the main panel has at least 3 available breaker slots. If the panel is full, a subpanel or panel upgrade will be needed and quoted separately. Drywall or cabinet cutting for wire access is included. Patching and painting are not included. Homeowner is responsible for supplying the recessed LED fixture prior to installation date. GFCI protection is required by NEC 2023 for all kitchen countertop outlets. This is a code compliance item. This estimate is valid for 30 days from date of issue. Payment Terms: 50% deposit required to schedule. Balance due upon job completion. Acceptance: Signing below constitutes agreement to the scope and price above. Customer Signature: ___________________________________________ Date: ______________

| PACIFIC COAST ELECTRIC Licensed Electrical Contractor ¦ CA Lic. #C10-892341 (408) 555-0147 ¦ info@pcelectricsj.com ¦ San Jose, CA 95123 | ESTIMATE Estimate No: EST-2025-004 Date Issued: March 10, 2025 Valid Until: April 9, 2025 |
| --- | --- |

| BILL TO Name: Robert & Patricia Delgado Phone: (408) 555-8822 Email: rdelgado1968@comcast.net Address: 6633 Santa Teresa Blvd, San Jose CA 95119 | JOB SITE / PROJECT Service Address: 6633 Santa Teresa Blvd, San Jose CA 95119 Job Type: Whole-Home Outlet & GFCI Safety Update |
| --- | --- |

SCOPE OF WORK Safety inspection and outlet upgrade for 1970s-built home. Replace 18 two-prong ungrounded outlets with modern 3-prong GFCI-protected outlets (using GFCI method per NEC 406.4 — no rewiring required). Replace 4 standard outlets in bathrooms with GFCI outlets. Replace 3 worn/cracked outlet covers. Install 2 new outdoor GFCI outlets (weatherproof covers) on front and rear of home. No permit required for straight outlet replacement.

LINE ITEMS

| Description | Qty | Unit | Labor ($) | Materials ($) | Line Total ($) |
| --- | --- | --- | --- | --- | --- |
| GFCI outlet (for ungrounded 2-prong replacement, 18 locations) | 18 | ea | 540.00 | 360.00 | 900.00 |
| GFCI outlet — bathroom locations (4 locations) | 4 | ea | 120.00 | 80.00 | 200.00 |
| Standard outlet + faceplate replacement (worn/cracked, 3 locations) | 3 | ea | 60.00 | 18.00 | 78.00 |
| Outdoor GFCI outlet with in-use weatherproof cover (2 locations, new circuit each) | 2 | ea | 300.00 | 160.00 | 460.00 |
| Miscellaneous — wire nuts, faceplates, GFCI test labels | 1 | lot | 0.00 | 28.00 | 28.00 |

|  | Labor Subtotal: $1,020.00 |
| --- | --- |
|  | Materials Subtotal: $646.00 |
|  | GRAND TOTAL: $1,666.00 |

TERMS & NOTES No permit is required for straight outlet replacement under current San Jose building code. Outdoor outlet locations require a short wire run from nearest interior circuit. Price includes up to 15 ft of wire per location. Longer runs quoted additionally. Per NEC 406.4(D), GFCI outlets can legally replace ungrounded 2-prong outlets without rewiring. Each replaced outlet will be labeled 'GFCI Protected / No Equipment Ground' per code. This estimate does not include any panel work, circuit additions, or aluminum wiring remediation. This estimate is valid for 30 days from date of issue. Payment Terms: 50% deposit required to schedule. Balance due upon job completion. Acceptance: Signing below constitutes agreement to the scope and price above. Customer Signature: ___________________________________________ Date: ______________

| PACIFIC COAST ELECTRIC Licensed Electrical Contractor ¦ CA Lic. #C10-892341 (408) 555-0147 ¦ info@pcelectricsj.com ¦ San Jose, CA 95123 | ESTIMATE Estimate No: EST-2025-005 Date Issued: March 12, 2025 Valid Until: April 11, 2025 |
| --- | --- |

| BILL TO Name: Amir Nazari Phone: (408) 555-6604 Email: amir.nazari@hotmail.com Address: 3318 Senter Road, San Jose CA 95111 | JOB SITE / PROJECT Service Address: 3318 Senter Road, San Jose CA 95111 (Detached Garage / ADU Conversion) Job Type: Detached Garage — ADU Conversion Electrical |
| --- | --- |

SCOPE OF WORK Full electrical buildout for a 480 sq ft detached garage being converted to an Accessory Dwelling Unit (ADU). Install 60A subpanel fed from main house panel (40 ft underground conduit run). Subpanel to serve: 8 standard 20A duplex outlets, 4 GFCI outlets (bathroom + kitchen area), 6 recessed LED fixtures with 3-way switching, 1 bathroom exhaust fan, 1 mini-split HVAC dedicated 30A circuit, 1 dedicated 20A kitchen circuit. Pull city permit. All work to NEC 2023 residential code.

LINE ITEMS

| Description | Qty | Unit | Labor ($) | Materials ($) | Line Total ($) |
| --- | --- | --- | --- | --- | --- |
| 60A Square D subpanel (12-space) + main breaker | 1 | ea | 0.00 | 185.00 | 185.00 |
| 60A 2-pole breaker in main house panel (feeder breaker) | 1 | ea | 0.00 | 55.00 | 55.00 |
| 4/3 aluminum feeder cable in 1.5" underground conduit (40 ft) | 40 | ft | 0.00 | 220.00 | 220.00 |
| Trenching and conduit burial (40 ft) | 40 | ft | 400.00 | 80.00 | 480.00 |
| Standard 20A duplex outlet (8 locations, 12/2 NM-B) | 8 | ea | 240.00 | 120.00 | 360.00 |
| GFCI outlet — bathroom + kitchen area (4 locations) | 4 | ea | 120.00 | 80.00 | 200.00 |
| Recessed LED fixture rough-in + 3-way switching (6 fixtures, 2 switch locations) | 6 | ea | 360.00 | 130.00 | 490.00 |
| Bathroom exhaust fan wiring + switch (fan supplied by owner) | 1 | ea | 90.00 | 25.00 | 115.00 |
| Mini-split HVAC — dedicated 30A / 240V circuit + disconnect | 1 | ea | 180.00 | 95.00 | 275.00 |
| Dedicated 20A kitchen circuit | 1 | ea | 120.00 | 45.00 | 165.00 |
| Misc — breakers, boxes, staples, connectors, wire nuts | 1 | lot | 0.00 | 110.00 | 110.00 |
| City of San Jose ADU electrical permit + plan check fee | 1 | ea | 0.00 | 350.00 | 350.00 |

|  | Labor Subtotal: $1,510.00 |
| --- | --- |
|  | Materials Subtotal: $1,145.00 |
|  | Permit & Inspection Fees: $350.00 |
|  | GRAND TOTAL: $3,005.00 |

TERMS & NOTES Homeowner is responsible for providing clear trench access path from main house panel to garage. Concrete cutting, if required, is not included and will be quoted separately. ADU electrical work requires a city permit and two inspections (rough-in and final). Electrician will schedule both inspections. Main house panel must have a minimum of 2 available breaker slots for the 60A feeder breaker. If full, a panel upgrade is required and will be quoted separately. Recessed fixtures supplied by homeowner prior to installation date. Fixture selection must be compatible with insulated ceiling application (IC-rated). This estimate is valid for 30 days from date of issue. Payment Terms: 50% deposit required to schedule. Balance due upon job completion. Acceptance: Signing below constitutes agreement to the scope and price above. Customer Signature: ___________________________________________ Date: ______________

TASK 0.1 — STUDY GUIDE

Now that you have reviewed all five estimates, here is every field that appeared across them — and why each one matters. Use this list to complete your Task 0.1 deliverable.

COACH NOTE — Task 0.1 Study Guide After reading all five estimates above, write down every field that appeared in more than one of them. Here is the full list — check how many you spotted on your own: Business Name + License Number: Every estimate identifies the contractor and their CA state license number. This builds trust and is legally required for work over $500 in California. Estimate Number: A unique ID for record-keeping and reference. When a customer calls to ask about their quote, both sides can find it instantly. Date Issued + Valid Until: Locks in pricing for a set period (usually 30 days). Materials and labor costs fluctuate, so electricians protect themselves with an expiry date. Customer Name, Phone, Email, Address: Basic contact information. The email is where the estimate gets sent — and where follow-up reminders go if they don't respond. Job Site / Service Address: May differ from billing address. Important for scheduling and for the permit application. Scope of Work: A plain-language description of exactly what will and won't be done. This prevents disputes. If it's not written here, it's not included. Itemized Line Items (Description, Qty, Unit): Each task broken into its own row. Customers trust itemized quotes more than a single lump-sum number — they can see what they're paying for. Labor vs. Materials split per line: Separating labor and materials on each line lets the customer see the true cost of each component. It also helps the contractor know their margins. Labor Subtotal + Materials Subtotal: Rolled up at the bottom so the customer (and contractor) can instantly see the proportion of the job that is labor vs. materials. Typically 60/40. Permit & Inspection Fees: A separate line because permits are a pass-through cost at actual cost — not marked up. Missing this is a common mistake that reduces profit. Grand Total: The single number the customer focuses on. Bold, prominent, no ambiguity. Payment Terms: Specifies deposit amount and when the balance is due. 50% deposit is standard for residential electrical work. Exclusions / Notes: What is NOT included is just as important as what is. Drywall patching, fixture supply, panel upgrades if needed — all explicitly excluded. Validity Period: Restated in the notes section to reinforce the urgency to decide. Most electricians use 30 days. Customer Signature Line: Makes the estimate a simple contract. Signing it means the customer agrees to the scope and price. These fields are the DNA of every estimate your AI tool will generate. The more clearly you understand what each one means and why it is there, the better your AI prompt will be in Phase 1.